ACC Terminal Emulator

Usage

accterm [com# [:i]] [br]

# 1-4 or a 3 digit hex port address

i IRQ for port 2-7

br baud rate 300-115200

Examples

"accterm 4800"	Uses COM1 at 4800 baud.

"accterm com2 4800" Uses COM2 at 4800 baud.

"accterm com5:2 4800 Uses non-standard COM5:IRQ2 4800 baud.

Notes

PatchMaster 200 requires 4800 baud for "Terminal" programming.

ACCTERM always forces 8/N/1 (8 data bits/no parity/1 stop bit).

<cr> = carrage return or enter.

Operation

ASCII keys typed are transmitted out the port.

Menu keys

Delete - ends program.

Insert - clears screen.

F1 - upload to Patchmaster.

F2 - download from Patchmaster.

Download from PatchMaster 200

Press T 7 1 F2 pm200.acc <cr> <cr> F2

Upload to PatchMaster 200

Press T 7 2 F1 pm200.acc <cr>