Paging Transmitters PM Checks

Revision 4004.1	Date 5/12/96

Models:

45 Watt    QT599X

250 Watt  QT-799X, GL-T8500

500 Watt   GL-T8600

C.S.

Techops Phone # 1-800-759-8324		Make no changes to station without Tech-Ops  approval

Readings outside normal values should be discussed with Tech-Ops for recomendations

Site Name/ Unit Id _______________________________________	Date: ____________

City: ___________________ St. ___________	Station Id __________  F1___   F2___

Model Transmitter _____________________ 	Serial Number ___________________

Model Reciever _______________________	RX Frequency ___________________

Data Circuit __________________________	Connected to ____________________

POTS phone number ___________________	Connected to ____________________

POTS phone number ___________________	Connected to ____________________

Shop Name ______________________________ 	Tech: __________________________

Goal and Intent: The PM check is intended to check the operation of a working station and to confirm / verify the station function and setup to meet or exceed the standards of  Skytel. The Pm check is not limited to the checks below. You should observe and generally address anything that could result in a decrease in reliability of the station operation.

Instructions:

Perform the PM inspection and fill out the data in the blanks provided. Note any problems in the system checkout. Make no changes to the site parameters and alignment without Tech-Ops Approval. Open work ticket to initiate a repair. Call Tech-Ops for approval prior to taking the station off the air. Advance notice to Tech-Ops of planned or scheduled work would greatly be appreciated.

Copy the requested information on the “Just the Facts FAX” and send or fax a copy of the 1 page data to the local TM for PM status reporting. Maintain the remainder of the worksheets / procedure for base line information in the station repair log to assist you in repairs until the next PM.

Test Equipment Required

Computer with terminal program N,8,1 9600

Serial Db9 cable

Bird Model 43 with 250 watt and 50 Watt element at specified freq operating range (Or equivelent)

6” RG 393 “N” test jumper or equivelent

Service Monitor with Spectrum Analyzer

Site Inspection						    		           Verify / Record #

Inspect Transmitter cleanliness and overall tidiness - Clean as necessarily

Check Rf Cables routing, are connectors tight, are cables secure.			_____

GPS antenna Mount - Inspect for security						_____

Station Back for loose wires and Wire routing.- Tie up loose wires			_____

Check Fans for operation, Verify Transient suppression and station grounding		_____

Any additional installation weakness, Note Exceptions on Comment section		_____

For more detail, Refer to Installation guide in Skytel Systems manual

Receiver Operational Checks

Connect Receiver Antenna input to Spectrum Analyzer

Set: Spectrum Analyzer - 20 khz per division and  for receiver frequency

Measure and record receive level 			Level _____Dbm

Measure and record baseline noise level                    Level_____Dbm

Set: Spectrum Analyzer -100 khz per division and for receive frequency

Are any carriers seen with in +/- 500khz  > than 40dbm above measured rx level ?_____

Receiver Frequency Check    (Skip this test if  RL72/73/903 Model Receiver)

Equipment Needed: Service Monitor, sniffer cable.

To construct Sniffer cable: Take 3-4’ piece of  RG-58 or similar cable with a BNC male 		connector on one end.  Strip back 3” of shield leaving center conductor insulator intact.

Connect sniffer cable to service monitor’s antenna input and place near receiver 	oscillator/crystal.

Set Service monitor to the Local Osc. frequency calculated from the chart below.

Record measured frequency error. ( If > than +/- 1.5 Khz call Tech-Ops,

adjustment may be required)                                                                       Freq error _______

| ttttjh Receiver | Operating Freq | IF Freq | Local Osc. Freq |
| --- | --- | --- | --- |
| Ttttt RL-70 | (plus ) | 11.2 Mhz. (=) |  |
| RL-451 | (minus) | 11.2 Mhz. (=) |  |
| RL-900 | (minus) | 10.7 Mhz (=) |  |

SINAD Measurements

Equipment Needed:   Signal Generator equipped with a SINAD option or external SINAD meter.

Connect  cable from Signal Generator to Receiver Input

Connect  Scope probe or suitable cable from SINAD input to rx audio high and ground on 	rear terminal of receiver:

| Connections | RL-451 | RL-70/72 | RL-900 |
| --- | --- | --- | --- |
| Audio High | TB1-7 | TB1-7 | TB2-1 |
| Audio Low | TB1-8 | TB1-8 | TB2-7 |

Record levels but make no adjustments without Techops Approval

Set signal generator to receive frequency and adj level to -65Dbm or 100uV

Set to generate 1khz tone with 3 khz deviation ( 2 Khz. Deviation if Narrow Band Rx)

Reduce generator output until a 12db SINAD reading is achieved and record level.

(If level is > .75uV or 108 Dbm call Tech-Ops, receiver repair /

alignment may be needed.                                                               Level _______Dbm

C2000									            Verify / Record #

Log onto C 2000 unit with password H2AWK

Enter the Version of Software							_____

Go to Status Screen (Menu item #1)

GPS lock (4 Satellites minimum)						_____

Estimated Time Error							_____

Modem Syncronized and Modem Recieving (YES)				_____

Link Bit Errors (should be 0) Call Tech-Ops if greater than 0	Record # Errors ;	_____

# active Alarms (Should be 0 when PM Completed)				_____

Exciter mode (On Line)							_____

Go to Link Input Configuration Screen (#4)

Check Link ID (Unique per Market)	Record	Link ID #	____________

Alternate Link Id (F1=111 F2 = 222)	Record	Link #		____________

Type = Analog 9600						____________

Link Channel = 1							_____

Modem Syncronized and Receiving = Yes					_____

Analog Signal to Noise ratio (Ideally >= 20 db)			___________dB

Contact Techops if <20 db could be RX problem and Repair may be necessary

Link Level = OK								_____

Go to Output Configuration Screen (#6)

Exciter Mode should be On Line					_____________

Exciter / pa delay (16us for PSL exciter, 4407 for DSP exciter, 0 for GL 4L)    _______

Note Simulcast Offset delay (defined by Skytel Plan)                                        _______

Go to Alarm Configuration Screen (#8)

Go to Analog Input Calibration (#2)

Verify that all Current Values fall within the programmed High and Low Limit	______

All other screens are programmed by Techops

Go to Syncronization Configuration Menu (#9)			            Verify / Record #

Syncronize to GPS = YES ( Contact Techops if  NO)				______

Estimated time Errors = 1 (If greater than 1 and or drifting Possible GPS Problem)____

Verify GPS Receiver Status = FULL ACCURACY				______

GPS Satellites = 4 or more satellites locked onto GPS (4 MIN)  # Satellites	______

Record Link/GPS Difference                                                                               ______

DSP Exciter

Note: DSP exciter Freq and Deviation is software controlled

Connect Computer to DSP exciter and Verify Programmed freq and Deviation on DSP

Read off the Main Status Screen

Read version of Software	Control ver ________	DSP ver_________

F1 Freq = 931.9375 for all 8 channels, F2 Freq = 931.4375 for all 8 channels

Record for all DSP Exciters                            Only Record for  85/8600 series transmitters

Deviation = 4500				FWD PWR  ___________ REV PWR __________

Operating channel = 1			Pre Amp      ___________ IPA            __________

Offset         ___________ hz		Driver 1      ___________  Driver 2     __________

VCO 1 voltage ____________v		Pa 1 A         ___________ Pa 1 B        __________

VCO 2 voltage ____________v		Pa 2 A         ___________ Pa 2 B        __________

Pa 3 A         ___________ Pa 3 B        __________

Pa 4 A         ___________ Pa 4 B        __________

Test Equipment Measurements

Install Wattmeter Onto Station Telewave Power Monitor or Isolator Output

Do not use test jumper longer than 6” to connect wattmeter.	  ______Watts

Observe Station Freq and Deviation on Service Monitor

Calibrate Front Panel Wattmeter and adjust total station output power

Front of station -Set PA Power output Pot for station rated ouput power on Front panel 					fwd power meter

Rear of  station - Adjust Telewave Power Monitor FWD power pot (AGC) for Station 					rated output power on Wattmeter installed at Power Monitor output 					connector.

_________ (check)

Verify Front Panel Meter Readings - Set Meter switch to MULTIMETER

Typical Values read with Station operating at Rated Power

Before Adj.     After Adj.

| Forward Power | Station Rated Power | _______ | _______ | Watts |
| --- | --- | --- | --- | --- |
| Reflected Power | 3->5 | _______ | _______ | Watts |
| Pre Amp | 200 -> 250 | _______ | _______ | ma |
| IPA | 1.75 -> 2.75 | _______ | _______ | Amps |
| Driver | 5-> 7 | _______ | _______ | Amps |
| Pa #1 | 5-> 7 | _______ | _______ | Amps |
| Pa #2 | 5-> 7 | _______ | _______ | Amps |
| Pa #3 | 5-> 7 | _______ | _______ | Amps |
| Pa #4 | 5 ->7 | _______ | _______ | Amps |

Note: All of the pa module current readings should be close to the same readings +- 1.0 amp

Connect Exciter output to Service Monitor to Measure TX Freq and Deviation

Skytel plan calls for 4500 hz deviation and F1 Freq = 931.9375 all 8 channels ,

F2 Freq = 931.4375 all 8 channels

Tx Freq___________Mhz

Deviation with Data __________ khz   This will not be a true reading but it should be

about 5 khz. Notify Tech-Ops if it is extreme.

PSL and 4L exciters - Refer to Manual for Freq / Deviation settings. Set Deviation to 4.5 khz

PSL exciters:

Sw1/3:- Modulate RF with paging data?

Connect Exciter Output to service monitor to measure freq error.

Change SW1/3 to Open  (Removes Modulation)

Measure Freq Error plus offset on Service Monitor	Error _____ Hz

Change SW1/3 to Closed  (station will modulate with paging data)

Sw2 - Modulation setting (4500 hz deviation setting)

SW2/1      Open

SW2/2      Closed

SW2/3      Open

SW2/4      Closed

Sw3 - Frequency offset setting switch

Report Offset    Read Switch bank 3     ___   ___   ___   ___

See table below for  carrier frequency offset  _____  hz.

Sw3/1	Sw3/2	Sw3/3	Sw3/4	Freq offset

Closed	Closed	Closed	Closed	-80 hz

Closed	Closed	Closed	Open	-70 hz

Closed	Closed	Open	Closed	-60 hz

Closed	Closed	Open	Open	-50 hz

Closed	Open	Closed	Closed	-40 hz

Closed   Open 	Closed	Open	-30 hz

Closed	Open	Open	Closed	-20 hz

Closed	Open	Open	Open	-10 hz

Open	Closed	Closed	Closed	0 hz	Factory Setting

Open	Closed	Closed	Open	+10 hz

Open	Closed	Open	Closed	+20 hz

Open	Closed	Open	Open	+30 hz

Open	Open	Closed	Closed	+40 hz

Open 	Open	Closed	Open	+50 hz

Open	Open	Open	Closed	+60 hz

Open	Open	Open	Open	+70 hz

Instructions for Checking Offset Frequency on PSL exciters

1. Connect Exciter output to Service monitor input (RF input connector 				     capible of accepting Exciter power)

2. Set SW1 switch #3 to Open (turns off paging data modulation)

3. Set SW3 switches 1 through 4 to Factory setting with 0 hz offset.

4. Read Frequency Error on Service monitor & note error.       _________hz

5. Change SW3 switches 1 through 4 to Skytel specified offset.

6. Read Frequency Error on Service monitor. Error will change from the

reading in step 4 by the amount of the offset programmed on SW3.

7. Change SW1/3 to Closed (Turns on Paging Data Modulation)

8. Connect Exciter output to PA input			Check ______

4 L Exciters

Connect Exciter Output to Service Monitor

Set: Data - Remote - Analog Switch to the Analog Position (Removes Deviation)

Measure  exciter frequency and note Freq Error...			Error _____hz

Set: Data - Remote - Analog Switch to the Data Position

(Station now deviates with Paging Data)

Connect Computer to C2000

Record levels but make no adjustments without Techops Approval

Go to Output Configuration Screen

MAKE SURE THE EXCITER OUTPUT IS CONNECTED TO TEST EQUIPMNET

Set Exciter mode to Test

Observe “STAIRSTEP” Waveform on Spectrum Analyzer Demodulated Waveform

CHECK_____

Set Deviation (#6) to:

| -100 % | - 33 % | + 33 % | + 100 % |
| --- | --- | --- | --- |
| LL | LL | LL | LL |

Reading	____

Adjustment = -Hi pot to TX freq - 4.5 khz (Compensate for Freq Error)

Set Deviation (#6) to:

| -100 % | - 33 % | + 33 % | + 100 % |
| --- | --- | --- | --- |
| LH | LH | LH | LH |

Reading ____

Adjustment = -Low pot to TX freq - 1.5 khz (Compensate for Freq Error)

Set Deviation (#6) to:

| -100 % | - 33 % | + 33 % | + 100 % |
| --- | --- | --- | --- |
| HH | HH | HH | HH |

Reading ____

Adjustment = +Low pot to TX freq + 1.5 khz (Compensate for Freq Error)

Set Deviation (#6) to:

| -100 % | - 33 % | + 33 % | + 100 % |
| --- | --- | --- | --- |
| HL | HL | HL | HL |

Reading  ____

Adjustment = +Hi pot to TX freq + 4.5 khz (Compensate for Freq Error)

Put paging back on line:

Set Deviation (#6) to:

| -100 % | - 33 % | + 33 % | + 100 % |
| --- | --- | --- | --- |
| LL | LH | HH | HL |

Observe “STAIRSTEP” Waveform on Spectrum Analyzer Demodulated Waveform

CHECK_____

Set Exciter mode to ON LINE

Connect Exciter output to PA input					CHECK____

Document Rf Devices Installed

Device				     Yes	     No

| Rx Spitter | ___ | ___ |
| --- | --- | --- |
| Rx Bandpass | ___ | ___ |
| Tx Combiner | ___ | ___ |
| Tx Bandpass /Notch Filter | ___ | ___ |
| Tripple Circulator | ___ | ___ |

Perform this step if the station has one of the above RF devices installed on Tx output

Connect Wattmeter to Cavity output (into Transmission line)      FWD _____REV_____  Watts

Compare cavity output to station output power (Cavity input verses Cavity output)

If techops instructs to tune cavity perform this procedure

Name of  Techops representative authorizing Cavity tune __________________

(Tuning cavity Using High Power may damage cavity!!!)

1. Connect Exciter ouptut to cavity input.

2. Connect Wattmeter to Cavity output

3. Terminate Wattmeter output into 50 ohm dummy load

4. Adjust cavity for maximum through-put

5. Connect Transmission line to Wattmeter output.

6. Connect Exciter to Pa input and measure Cavity Output Power    _____Watts

7. Remove Wattmeter and reconnect RF cables to normal paging connections

Log onto C2000

Verify No active alarms.

Clear Alarm history

Verify that Battery Backup Unit is in the on position

Remove test equipment and verify station is paging.

Upon completion of procedure contact Tech-Ops prior to leaving site.

Report PM data to TM for Data base update.

Problems Noted:_______________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

Comments:____________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

Just the Facts Fax

Paging Transmitter PM Data

TM Name: ________________________________________ PM Date: ____________

Site Name _______________________________________	Unit Id ________

City: ___________________ St. ___________	Site Id __________  F1___   F2___

Model Transmitter _____________________ 	Serial Number ___________________

Model Reciever _______________________	RX Frequency ___________________

Data Circuit __________________________	Connected to ____________________

POTS Phone Number ___________________	Connected to ____________________

POTS Phone Number ___________________	Connected to ____________________

Shop Name ______________________________ 	Tech: __________________________

Shop Phone #:__(_____)________-_____________

Reporting Data

| Readings | ____ | Wattmeter FWD power | _________ |  |
| --- | --- | --- | --- | --- |
| C2000 Software Ver | ____ | Wattmeter REV power | _________ |  |
| Link ID # | ____ | Measurements: |  |  |
| DSP Control Version | ____ | Rx Sens. (12dB Sinad) | _______dB |  |
| DSP Version | ____ | Site Floor Noise | _______dB |  |
| Alt Link ID# | ____ | Rx Frequency Error | _______hz |  |
| Link Bit Errors | ____ | Station Output power | _______W |  |
| Analog S/N Ratio | ____ | Cavity output power | _______W |  |
| Exciter Delay | ____ | Data Deviation observed | _______khz |  |
| Simulcast Offset Delay | ____ | Offset Delay inserted | ________hz |  |
| # Satellites GPS is locked on | ____ | Front Panel Meter readings |  |  |
| Syncronized to GPS. Y/N? | ____ | FWD Power | _______W |  |
| Type of Exciter (Dsp/PSL/4L | ____ | REV Power | _______W |  |
| Exciter PA Delay (4407/16/0) | ____ | Pre Amp | _______A |  |
| DSP VCO1 voltage | ____ | IPA | _______A |  |
| DSP VCO2 voltage | ____ | Driver | _______A |  |
| Repair Initiated Y/N? | ____ | PA #1 ( A, B) | _______A | _______A |
| IF yes Work Ticket Number | ____ | PA #2 (A, B) | _______A | _______A |
|  |  | PA #3 (A, B) | _______A | _______A |
|  |  | PA #4 (A, B) | _______A | _______A |

Document Rf Devices Installed

Device				     Yes	     No

| Rx Spitter | ___ | ___ |
| --- | --- | --- |
| Rx Bandpass | ___ | ___ |
| Tx Combiner | ___ | ___ |
| Tx Bandpass /Notch Filter | ___ | ___ |
| Tripple Circulator | ___ | ___ |

Comments:__________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________