Rich’s Home Made Repeater

A couple of years ago a friend of mine had the idea to put a six meter repeater on the air.  I went along with the idea and a few other friends pitched in.  The most expensive piece of equipment was the controller.  Sad to say we were on the air for less than a year but I got the experience with interfacing repeater components and had a lot of fun.  I have since become a better computer programmer than I was then and I decided to write a simple program for repeater operation.  Please feel free to use the following information free of charge and contact me with any questions or if you need to request new software.

The program repeater.exe is a program that will change an old model computer into a repeater controller.  It will work with the older versions of DOS up through Windows 98se.

The file morse.txt is a text file that the program uses to send the repeater ID to the tone generator.

The DTMF.doc file is a schematic that has been tested and works well on some computers.  If you use this schematic and find it does not work on your computer check the voltage between pin 17’s connection to the CD22204 chip and ground.  On some computers I have found that the voltage drops below the required 4 volts to run the chip.  If this happens then disconnect the chips from pin 17 and use a regulated 5 volts to power the chips in its place.  The +5 volts (VDD) should be wired in place of pin 17 (PPT) and the ground (VSS) connected to the grounded pins on the printer port (PPT).

The repeater.rnf file is a data file used by the repeater.exe program. Do not attempt to open this file. If it is opened it may become corrupted and the program will not work.

If you want tone access required on your repeater I recommend using a TS-64.  I used one and it will provide a ground (COR) for pin 11.  If you use a TS-64 then you will not need the LM339 chip.  The TS-64 can be found from AES and other Amateur Suppliers.

The program REPEATER.EXE is only designed to do a Morse code ID.  I am currently working on a program that will do voice ID and announcements using the computers sound card.  It will also include an interface for a link radio and remote control functions of other site equipment.  Below is a wiring chart that may simplify the interface between the printer port and the radio interfacing circuit.  I have tested this software and it will not work on later versions of windows.  It works very well with versions up to windows 98se and later DOS versions. You can make a bootable 3.5 disk and put the program on there and it will work on a computer without a hard drive.

Rich’s Home Made Repeater

# PPT Wiring Chart

Printer port wires              Interfacing with equip

------------------              ----------------------

pin # 11                         RX Detection (COR)

pin # 15                         CD22204 pin 2

pin # 13                         CD22204 pin 1

pin # 12                         CD22204 pin 14

pin # 10                         CD22204 pin 13

pin # 2                           tone (CW) generator

pin # 6                           TX PTT

pins 21-25                     Common Ground

The PPT (pin # 11) needs to go to ground when a signal is received on the RX Radio.  Pins 10,12,13 and 15 will go to the DTMF CD22204 Chip pins as shown above.

Pin # 2 will supply a high (around 3 volts on most computers) and should go to a tone generator for the Morse Code ID.  It can also be hooked to a LED in series with a 220 ohm resister that will flash in code the call sign on the repeater.  Pin # 6 will supply a high to key the transmitter (PTT).  This can be done simply with a dc relay or preferably a comparator (LM339) or possibly a transistor.

You can choose from many different types of tone generators.  If you want a lot of options you can use a basic stamp.  The least expensive is probably a 555 Timer.  Below is an example of using a 555 Timer chip to generate Morse code for your repeater ID.  This was copied off the net.  There are many varieties of tone generators on the net.

Rich’s Home Made Repeater

In the above 555 Timer example you need to put a transistor between the 555 timer pin 1 and ground.  Put the collector on 555 Timer pin 1 and the emitter to ground. Pin 2 from the printer port (PPT) will go to the base of the transistor.  You may need to put some diodes or resistance in line between (PPT) Pin 2 and the base to get the voltage on the base less than .7 volts.

On some schematics there is a variable resistor that will change the tone frequency.  The 4.5 volts can come from your (PPT) pin 17 or a regulated power supply.  Ground of course is common through out the circuit and should be connected to the (PPT) pins 20 – 25.  From the 8 ohm speaker you can forward the + to the microphone input + of your transmitter.  Your microphone input – is of course common to ground.

Note:  You can use a quad comparator LM339 chip as a Solid State relay for the PTT, and to signal the tone generator as well as detect a signal on the RX (COR).

Feel free to Email me if you have any questions or suggestions.

pixlea@peoplepc.com

73 KE4JFV

*NOTE:  Please don’t hesitate to email me with any questions you may have.  Send me your ideas to improve this Home Made Repeater.  Simple modifications to the software to meet your special needs can be requested as well.