Rich’s Home Made Repeater

The above schematic will work on some computers and has an advantage above the others because

it does not require an outside voltage source to power the interface.  To test if this schematic will work on

your computer use a fluke multi-meter and read the following voltages while the repeater control program

is running.  This schematic was designed and tested using an old Tradewin Computer and Windows 98.

PPT Pin 17 has to read at least 5 volts, no more than 7 volts.

PPT Pin 3 needs to read less than the voltage on the speaker output of the RX.

Example: On the above circuit the voltage on the radio speaker out was a little over 4 volts when receiving a signal and less than 1 volt when squelched silent. Pin 3 was just below 3 volts. This allows the LM339 Comparator Chip to signal the computer repeater program when a signal is received.

I have found the CD22204 on the net for around $4.50 with the crystal. The other

parts can be purchased at your local electronics store for just a few bucks.

The above schematic is for the receive section only. Other example schematics are

available. Questions and suggestions are welcome. Email me no phone call please.